This DIY is about making your fogs as DRL. It is really simple, takes about 5 minutes.

What you need: 
-Flat screw driver
-Electrical tape
-Sharp knife

First, remove your headlight switch. 
Next, cut the yellow wire on slot 15 about 2 inches long from the switch.

Strip the wire on slot 13 for about ¼ inch long and connect the yellow wire to the #13 wire.

Secure all this with electrical tape

You are now done, reinstall everything. When you turn ignition on, automatically, the fogs will turns on.

I take no responsibility for any damage this modification to your electrical system may cause.