mk3 jetta/golf joey mod diy:

i just did this yesterday and obviously quite a few you asked for a DIY, i'll do the best i can to provide for you guys.

you will need the following:
-utility knife
-black primer
-black spray paint
-clear coat (your choice really)
-silicone sealent
-masking tape
-an oven that works properly
-calm music (your choice really)
-and most of all, patience

step one:
-remove grille

-remove headlamps (3 or 4 8mm bolts)

step two:
-preheat oven to between 185-200 degrees F (i personally tried it at around 190 and it worked but tried 200 and seemed to do a bit better)

-after its finished preheated stick the headlamp in the oven (do not let the lense touch any part of the oven)

-let it sit for for around 60-90 seconds (if that doesn't work well go with around 2 minutes, but no longer!)

step three:
-take out the headlamp and use the utility knife and start working away the silicone (once i had most of it removed all around i used a knife and easily pryed it open to where i could fit an unused pencil to keep that open a little bit and worked my way around the whole headlamp)

*the lense should come off a bit easier doing it that way

-repeat step two and three for the other headlamp

*BE SURE AND TAKE YOUR TIME WHILE DOING STEP THREE!

step four:
-after the lense is removed get your newspaper, black primer, black spray paint (or color of choice), clear coat, and masking tape

-use the masking tape to cover the area you do not want to be black, or you can find a more simple way of doing it go for it

-apply the black primer

-after the primer has dried apply as many coats of black paint as you'd like (i used 2 coats per headlamp to be safe)

-after your last coat of black paint has dried, apply the clear coat (i only used one coat of clear)

-once its completely finished and nice and dry, remove the masking tape

-repeat step four for the other headlamp

*while waiting for them to finish drying i cleaned the lenses inside and out with some window washing spray stuff and got them looking more clear

step five:
-once you've finished letting the paint dry apply the silicone sealent all the way around and place the lense on and make sure its placed correctly.

*if some of the silicone comes up on the sides just leave it until it has completely dried and use the utility knife to cut the extra off the sides.

*allow the silicone 24 hours to make sure its finished and is completely sealed off.

step six:
-once everything is completely finished drying and secure, put the headlamps back on and then the grille.

BE SURE AND TAKE YOUR TIME THROUGH THE WHOLE PROCESSS! it does take time but is well worth it. i will not be held accountable for anything followed in this diy, gots to put that

but, good luck, have fun, be safe, and keep on dubbin'. you can pm me if you have any questions whatsoever and i'll do my best to help you out!
peace - james n.