jus did mine today in liek 30 min. after reading on here that it takes upwards to 2 hrs here is how

disconnect battery

make sure steering wheel is locked

remove the steering column surround ( 2 screws on either end )

go under dash and using a 13mm wrench and socket remove the bolt holding the steering column to the u joint

unplug every plug goin into the steering column by the steering wheel including the ignition and airbag

now on the left side there is a gold round headed bolt that holds the column to the dash support. use a vise grip and take that out. comes out pretty easily after the first initial turn. can do it with your fingers after.. then u can pull the steering wheel and everything comes out as one piece.

now the only problem now is ur are goin to need a thin screw driver to reach the screw holding the ignition switch in.. after than just re install in reverse order and your done..

Do at your own risk though.. im not responsible if someone damages there car in the process or gets hurt this is jus a lil help for those who dont feel like pulling apart everything..