Decided to rotate tires on the B4 recently. While attempting to loosen the passengers front wheel lug bolts, I discovered one bolt with a head stripped so bad that you could turn a 17mm 6pt. socket on it and not feel any of the six points catching. I suspect this is one reason I got such a good price on the car.

Time to break out the drill with a 1/2 inch chuck. First bit used was 3/8 inch. Hole was drilled deep enough to be below the ball seat of the lug bolt (ball seat is what contacts the wheel). Once that was done, a 1/2 inch bit and cutting oil was used to drill through the head of the bolt. When the larger bit got just below the ball seat, the lug head gave way and came off with the bit (sigh of relief). Apparently the later (97) bolts I used as replacements had deeper recess than the OE (95) lug bolts.

After the wheel was removed, the remaining broken stud could be removed from the hub by hand.

Next set of tires you have put on your VR6, do yourself a favor and request that the lug bolts be tightened by hand (not to ~120lbft with a 1/2 inch drive impact wrench ).